DOVDE

IT GURU
11. Laboratorijska vježba –

Izvornik: L.L. Peterson & B.S. Davie, Computer Networks - A Systems Approach, 4ed., Elsevier, Inc., ISBN-13: 978-0-12-370548-8, 2007.

Sadržaj

# Laboratory - RSVP: Resource Reservation Protocol (Providing QoS by Reserving Resources in the Network)

## Objective

The objective of this lab is to study the Resource Reservation Protocol (RSVP) as a part of the Integrated Services approach to providing Quality of Service (QoS) to individual applications or flows.

## Overview

For many years, packet-switched networks have offered the promise of supporting multimedia applications, that is, those that combine audio, video, and data. Audio and video applications are examples of real-time applications. In the best-effort model, the network tries to deliver your data but makes no promises and leaves the "cleanup operation" to the edges. This model is not sufficient for real-time applications. What we need is a new service model - one in which applications that need better assurances can request such service from the network. The network may then respond by providing an assurance that it will do better or perhaps by saying that it cannot promise anything better at the moment. A network that can provide different levels of service is often said to support QoS.

Two approaches have been developed to provide a range of QoS: Integrated Services and Differentiated Services. The Resource Reservation Protocol follows the Integrated Services approach, whereby QoS is provided to individual applications or flows. The Differentiated Services approach provides QoS to large classes of data or aggregated traffic.

While connection-oriented networks have always needed some sort of setup protocol to establish the necessary virtual circuit state in the routers, connectionless networks like the Internet have had no such protocols. One of the key assumptions underlying RSVP is that it should not detract from the robustness that we find in the Internet. Therefore, RSVP uses the idea of soft state in the routers. Soft state - in contrast to the hard state found in connection-oriented networks—does not need to be explicitly deleted when it is no longer needed instead, it times out after some fairly short period if it is not periodically refreshed. RSVP adopts the receiver-oriented approach - the receivers keep track of their own resource requirements, and they periodically send refresh messages to keep the soft state in place.

In this tab you will set up a network that carries real-time applications and that utilizes RSVP to provide QoS to one of these applications. You will study how RSVP contributes to the performance of the application that makes use of it.

## Prelab Activities

- Read section 6.5 from "Computer Networks: A Systems Approach", 4th Edition.

Go to www.net-seal.net/animations.php and play the following animation:

- TCP Flow Control.

## Procedure

### Create the Project

- Start OPNET IT Guru Academic Edition  Choose Open from the File menu.

- Select the project you created in Lab 9: <your initials>_Queues  Click OK.

- From the File menu, choose Save As  Rename the project to <your initiate>_RSVP  Click OK.

- From the Scenarios menu, choose Manage Scenarios  Click on FIFO  Click Delete  Click on PQ  Click Delete.

- Click on WFQ and rename it to QoS_RSVP  Click OK.

- Make sure that you have only the QoS_RSVP scenario in your project. The following figure shows one way to check for the available scenarios in the project.

- Save your project.

The idea of the FQ (fair queuing) discipline is to maintain a separate queue for each flow currently being handled by the router. The router then service these queue in a round robin manner.

WFQ allows a weight to be assigned to each flow (queue). This weight effectively controls the percentage of the link’s bandwidth each flow will get. We could use the ToS (Type of Service) field in the IP header to identify that weight.

### Configure the Network

#### Add More VoIP Nodes:

In this project we will set up the two VoIP nodes so that one will always be the Caller party and the other will be the Called party. In addition, we will add two new VoIP Caller and Called nodes. These new nodes will utilize RSVP to reserve their required resources through the network.

- Right-click on the VoIP East node  Edit Attributes.  Rename the node to Voice Called  Assign None to the Application: Supported Profiles attribute  Assign Voice Called to the Client Address attribute  Click OK.

- Right-click on the VoIP West node  Edit Attributes.

  - Rename the node to Voice Caller.

  - Assign None to the Application: Supported Services attribute.

  - Edit the value of the Application: Destination Preferences attribute  Set Rows to 1  Assign Voice Destination to the Symbolic Name of the new row  Edit the Actual Name attribute  Set Rows to 1  Assign Voice Called to the Name attribute of the new row as shown.

  - Click OK three times.

- Click on the Voice Called node to select it  From the Edit menu, select Copy  From the Edit menu, select Paste (alternatively, use the standard keyboard shortcuts, Ctrl-C and Ctrl-V).

- Locate the new node somewhere below the Voice Called node on the screen  Connect the new node to the East Router using a 10BaseT link.

- Right-click on the new node  Edit Attributes.

- Click on the ethernet_wkstn value of the model attribute  Select Edit  Select the ethernet_wkstn_adv model.

- Rename it to Voice_RSVP Called  Assign Voice_RSVP Called to its Client Address attribute.

- Click OK.

- Copy and paste the Voice Caller node.

- Locate the new node somewhere below the Voice Caller node  Connect the new node to the West Router using a 10BaseT link.

- Right-click on the new node  Edit Attributes.

- Click on the ethernet_wkstn value of the model attribute  Select Edit  Select the ethernet_wkstn_adv model.

- Rename it to Voice_RSVP Caller.

- Edit the Application: Destination Preferences attribute  Open the Actual Name table by clicking in the value field of Actual Name  Assign Voice_RSVP Called to the Name attribute (this is to replace the current value, which is Voice Called).

- Click OK three times.

- Rename the Queues node in the project to QoS. Your project should look like the following diagram.

- Save your project.

| Applications | Profiles | QoS |
| --- | --- | --- |

#### Define the Data Flow:

Here, we will define the data flow characteristics of the voice traffic in the network. The sender's RSVP module periodically sends RSVP Path messages, that uses the data flow characteristics to describe the traffic generated by the sender. When the receiver's RSVP module receives the Path message, the receiver host application checks the characteristics of the requested data flow and decides if resources should be reserved. Once a decision is made to request network resource reservation, the host application sends a request to the local RSVP module to assist in the reservation setup. The receiver's RSVP module then carries the request as Resv messages to all nodes along the reverse data path to the sender.

The flow is defined by its required bandwidth and buffer size. Bandwidth is set to be the token bucket rate in the flow specification of the Path and Resv messages. The buffer size represents the amount of the application "bursty" data to be buffered. It specifies the token bucket size that will be set in the Path or Resv messages for the session.

      - Right-click on the QoS node  Edit Attributes.

- Expand the RSVP Flow Specification hierarchy and its row 0 hierarchy  Set Name to RSVP_Flow  Assign 50,000 to the Bandwidth (bytes/sec) attribute  Assign 10,000 to the Buffer Size (bytes) attribute.

- Expand the RSVP Profiles hierarchy and its row 0 hierarchy  Set Profile Name to RSVP_Profile.

- Click OK and then save your project.

#### Configure the Application:

Here we will create a VoIP application that utilizes the RSVP flow specifications we configured.

      - Right-click on the Applications node  Edit Attributes  Expand the Application Definitions hierarchy  Set rows to 4 (to add a fourth row to the Application Definitions attribute).

          - Name and set the attributes of row 3 as shown:

          - Click on the PCM Quality Speech value (shown above)  Select Edit  Edit the value of the RSVP Parameters attribute  Assign the following values (recall that we defined the RSVP_Flow in the QoS node)  Click OK three times.

Note that the characteristics of the Outbound Flow are carried in the Path messages to be sent from sender to receiver, and the characteristics of the Inbound Flow parameters are carried in the Resv messages to be sent from the receiver to the sender.

#### Configure the Profile:

- Right-click on the Profiles node  Edit Attributes  Expand the Profile Configuration hierarchy  Set rows to 4 (to add a fourth row to the Profile Configuration attribute)  Name and set the attributes of row 3 as shown:

- Click OK and then save your project.

#### Configure the Interfaces:

OPNET IT Guru supports RSVP on a per-interface basis; RSVP can be enabled or disabled for each node's interface.

- Simultaneously select (shift + left-click) the following three links:

- From the Protocols menu, select RSVP  Select Configure Interface Status  Make the selections shown below in the configuration dialog box  Click OK and then save your project.

The above process enables RSVP on all interfaces along the path between the two Voice parties that need to utilize RSVP.

#### Configure the Hosts and Routers:

In OPNET IT Guru, the RSVP process runs only in IP-enabled nodes. The advanced versions (*_adv) of those node models must be used, as we did already, to configure RSVP-related parameters. In addition, the RSVP model in OPNET IT Guru requires either WFQ or custom queuing schemes.

- Right-click on the Voice_RSVP Caller node  Edit Attributes.

- Expand the Application: Supported Profiles hierarchy and its row 0 hierarchy  Assign VolP_RSVP Profile to the Profile Name attribute.

- Expand the Application: RSVP Parameters hierarchy  Expand its Voice hierarchy  Enable the RSVP Status  Expand the Profile List hierarchy  Assign to the Profile attribute of row 0 the value RSVP_Profile.

- Expand the IP Host Parameters hierarchy  Expand its Interface Information hierarchy  Expand the QoS Information hierarchy  Assign WFQ to the Queuing Scheme attribute  Assign ToS Based to the Queuing Profile attribute  Assign RSVP Enabled to the RSVP Info attribute.

Type of service (ToS) is assigned to the IP packets. It represents a session attribute that allows packets to be provided the appropriate service in the IP queues.

- Expand the RSVP Protocol Parameters hierarchy  Expand the Interface Information hierarchy. (You should notice that the word Enabled is listed in the summary line. When you expand it, you will see that it is the value of RSVP Status. If Enabled is not listed, go back to the Configure the Interfaces steps.)  Expand the hierarchy of the row of that interface  Assign 75% to both the Maximum Reservable BW and Maximum Bandwidth Per Flow attributes as shown:

- Click OK.

Maximum Reservable BW specifies the percentage of the bandwidth of the connected link that RSVP can reserve on the interface.

Maximum Bandwidth Per Flow specifies the amount of reservable bandwidth that can be allocated to a single flow.

- Right-click on the Voice_RSVP Called node  Edit Attributes.

- Edit the Application: Supported Services attribute. The Application: Supported Services Table will popup  In that table, replace the VoIP Application with VolP_RSVP and Click OK.

- Expand the Application: RSVP Parameters hierarchy  Expand its Voice hierarchy.  Enable the RSVP Status  Expand the Profile List hierarchy  Edit the value of the Profile attribute of row 0 and enter RSVP_Profile.

- Expand the IP Host Parameters hierarchy  Expand its Interface Information hierarchy  Expand the QoS Information hierarchy  Assign WFQ to the Queuing Scheme attribute  Assign ToS Based to the Queuing Profile attribute  Assign RSVP Enabled to the RSVP Info attribute.

- Expand the RSVP Protocol Parameters hierarchy  Expand the Interface Information hierarchy. (You should notice that the RSVP Status of the interface that is connected to the router is Enabled. If not, go back to the Configure the Interfaces steps.)  Expand the hierarchy of the row of that interface  Assign 75% to both Maximum Reservable BW and Maximum Bandwidth Per Flow attributes.

- Click OK.

- Right-click on the East Router node  Edit Attributes.

- Click on the Ethernet4_slip8_gtwy value of the model attribute.  Select Edit  Select the Ethernet4_slip8_gtwy_adv model.

- Expand the RSVP Protocol Parameters hierarchy  Expand the Interface Information hierarchy. (You should notice that the RSVP Status of two interfaces, which are connected to the West Router and the Voice_RSVP Called node, are Enabled. If not, go back to the Configure the Interfaces steps.)  Expand the hierarchies of the rows of these two interfaces  Assign 75% to both Maximum Reservable BW and Maximum Bandwidth Per Flow attributes.

- Expand the IP Routing Parameters hierarchy  Expand the Interface Information hierarchy  Expand the hierarchies of the rows of the same two interfaces you configured in the previous step (step ii)  Expand the QoS Information hierarchy for both  Set Queuing Scheme to WFQ and Queuing Profile to ToS Based for both.

- Click OK.

- Right-click on the West Router node  Edit Attributes.

- Click on the Ethernet4_sIip8_gtwy value of the model attribute  Select Edit  Select the Ethernet4_sIip8_gtwy_adv model.

- Expand the RSVP Protocol Parameters hierarchy  Expand the Interface Information hierarchy. (You should notice that the RSVP Status of two interfaces, which are connected to the East Router and the Voice_RSVP Caller node, are Enabled. If not, go back to the Configure the Interfaces steps.)  Expand the hierarchies of the rows of these two interfaces  Assign 75% to both Maximum Reservable BW and Maximum Bandwidth Per Flow attributes.

- Expand the IP Routing Parameters hierarchy  Expand the Interface Information hierarchy  Expand the hierarchies of the rows of the same two interfaces you configured in the previous step (step ii)  Expand the QoS Information hierarchy for both  Set Queuing Scheme to WFQ and Queuing Profile to ToS Based for both.

- Click OK.

### Choose the Statistics

We will select statistics from three different nodes:

#### Voice_RSVP Caller Statistics:

- Right-click on the Voice_RSVP Caller node and select Choose Individual Statistics from the pop-up menu.

- Expand the RSVP hierarchy and select Number of Path States.

- Right-click on the Number of Path States statistic  Select Change Draw Style from the pop-up menu  Choose bar chart.

- Right-click on the Number of Path States statistic  Select Change Collection Mode from the pop-up menu  Check the Advanced checkbox  From the Capture mode drop-down menu, select all values, as shown  Click OK.

- Expand the Voice Calling Party hierarchy and select the following statistics: Packet Delay Variation and Packet End-to-End Delay (sec).

- Click OK.

Packet Delay Variation is the variance among end-to-end delays for voice packets received by this node.

Packet End-to-End Delay for a voice packet is measured from the time it is created to the time it is received.

#### Voice_RSVP Called Statistics:

- Right-click on the Voice_RSVP Called node and select Choose Individual Statistics from the pop-up menu.

- Expand the RSVP hierarchy and select Number of Resv States.

- Right-click on the Number of Resv States statistic  Select Change Draw Style from the pop-up menu  Choose bar chart.

- Right-click on the Number of Resv States statistic  Select Change Collection Mode from the pop-up menu  Check the Advanced checkbox  From the Capture mode drop-down menu, select all values  Click OK.

- Click OK.

#### Voice Caller Statistics:

- Right-click on the Voice Caller node and select Choose Individual Statistics from the pop-up menu.

- Expand the Voice Calling Party hierarchy and select the following statistics: Packet Delay Variation and Packet End-to-End Delay (sec).

- Click OK.

### Configure the Simulation

Here, we need to configure the duration of the simulation:

- Click on  and the Configure Simulation window should appear.

- Make sure that the duration is set to 150 seconds.

- Click on the Global Attributes tab and make sure that the following attribute is enabled:

        - RSVP Sim Efficiency = Enabled. This decreases the simulation time and memory requirements by not sending refresh messages (i.e., Path and Resv refreshes).

- Click OK and then save your project.

### Run the Simulation

To run the simulation:

- Click on  and then click the Run button. Depending on the speed of your processor, this may take several minutes to complete.

- After the simulation completes, click Close.

- Save your project.

### View the Results

To view and analyze the results:

- Select View Results from the Results menu.

- As shown in the following figure, choose the Packet End_to_End Delay for both the Voice Caller and Voice_RSVP Caller nodes. Choose Overlaid Statistics and time_average.

- Click Show to get the following graph. (Note: To zoom in on the graph, click and drag your mouse to draw a rectangle around the area of interest and release the mouse button.)

- Similarly, you can get the following graph that compares the Packet Delay Variation for both the Voice Caller and Voice_RSVP Caller nodes. (Note: Make sure to "unselect" the statistics you chose for the previous graph.)

- Finally, prepare the graph that displays the number of Path and Resv states by selecting the following statistics. Make sure to select Stacked Statistics and As Is as shown.

- Right-click on the resulting graph and choose Edit Panel Properties  Change the assigned values to the Horizontal Min and Horizontal Max fields as shown (your graph might require a slightly different range):

- Click OK. The resulting graph should resemble the one below.

## Further readings

- OPNET RSVP Model Description: From the Protocols menu, select RSVP  Model Usage Guide.

## Exercises

- Analyze the graphs we obtained in this lab. Show the effect of RSVP on the Voice application and explain the obtained numbers of Path and Resv states.

- How does the data rate of the link connecting the East and West routers affect the performance (e.g., Packet End-to-End Delay) of the Voice and Video Conference applications? To answer this question, create a new scenario as a duplicate of the QoS_RSVP scenario. Name the new scenario Q2_HighRate. In the Q2_HighRate scenario replace the current PPP_DS1 link (data rate 1.544 Mbps) with a PPP_DS3 link (data rate 44.736 Mbps).

## Lab Report

Prepare a report that follows the guidelines explained in Lab 0. The report should include the answers to the above exercises as well as the graphs you generated from the simulation scenarios. Discuss the results you obtained and compare these results with your expectations. Mention any anomalies or unexplained behaviors.