DOVDE

Laboratory - Token Ring (A Shared-Media Network with Media Access Control)

IT GURU
6. Laboratorijska vježba – Protokol usmjeravanja informacija - RIP

(na temelju algoritma vektora udaljenosti)

Izvornik: L.L. Peterson & B.S. Davie, Computer Networks - A Systems Approach, 4ed., Elsevier, Inc., ISBN-13: 978-0-12-370548-8, 2007.

Sadržaj

# Laboratory - RIP: Routing Information Protocol (A Routing Protocol Based on the Distance-Vector Algorithm)

## Objective

The objective of this lab is to configure and analyze the performance of the Routing Information Protocol (RIP) model.

## Overview

A router in the network needs to be able to look at a packet's destination address and then determines which one of the output ports is the best choice to get the packet to that address. The router makes this decision by consulting a forwarding table. The fundamental problem of routing is: How do routers acquire the information in their forwarding tables?

Routing algorithms are required to build the routing tables and hence forwarding tables. The basic problem of routing is to find the lowest-cost path between any two nodes, where the cost of a path equals the sum of the costs of all the edges that make up the path. Routing is achieved in most practical networks by running routing protocols among the nodes. The protocols provide a distributed, dynamic way to solve the problem of finding the lowest-cost path in the presence of link and node failures and changing edge costs.

One of the main classes of routing algorithms is the distance-vector algorithm. Each node constructs a vector containing the distances (costs) to all other nodes and distributes that vector to its immediate neighbors RIP is the canonical example of a routing protocol built on the distance-vector algorithm. Routers running RIP send their advertisements regularly (e.g., every 30 seconds). A router also sends an update message whenever a triggered update from another router causes it to change its routing table.

The Internet Control Message Protocol (ICMP) can be utilized to analyze the performance of the created routes. It can be used to model traffic between routers without the need of running applications in an end node.

In this lab you will set up a network that utilizes RIP as its routing protocol. You will analyze the routing tables generated in the routers, and you will observe how RIP is affected by link failures. You will also utilize the ICMP to create echo reply messages (i.e., ping) to analyze the created routes.

## Prelab Activities

- Read sections 4.1.5, 4.1.7, and 4.2.2 from "Computer Networks: A Systems Approach", 4th Edition.

Go to www.net-seal.net/animations.php and play the following animations:

- The Address Resolution Protocol (ARP). Example1, Example2, Example3.

- ARP with Multiple Networks.

- Routing and Forwarding.

## Procedure

### Create a New Project

- Start OPNET IT Guru Academic Edition  Choose New from the File menu.

- Select Project and Click OK  Name the project <your initials>_RIP, and the scenario NO_Failure  Click OK.

- In the Startup Wizard: Initial Topology dialog box, make sure that Create Empty Scenario is selected  Click Next  Select Campus from the Network Scale list  Click Next three times  Click OK.

### Create and Configure the Network

#### Initialize the Network:

- The Object Palette dialog box should now be on top of your project workspace. If it is not there, open it by clicking Error! Objects cannot be created from editing field codes.. Make sure that the internet_toolbox is selected from the pull-down menu on the object palette.

- Add to the project workspace the following objects from the palette: one ethernet4_slip8_gtwy router and two 100BaseT_LAN objects.

- To add an object from a palette, click its icon in the object palette  Move your mouse to the workspace  Click to place the object  Right-click to stop creating objects of that type.

- Use bidirectional 100BaseT links to connect the objects you just added as in the following figure. Also, rename the objects as shown (right-click on the node  Set Name).

- Close the Object Palette dialog box.

- Save your project.

The ethernet4_slip8_gtwy node model represents an IP based gateway supporting four Ethernet hub interfaces and eight serial line interfaces. IP packets arriving on any interface are routed to the appropriate output interface based on their destination IP address. The Routing Information Protocol (RIP) or the Open Shortest Path First (OSPF) protocol may be used to dynamically and automatically create the gateway’s routing tables and select routes in an adaptive manner.

#### Configure the Router:

- Right-click on Router1  Edit Attributes  Expand the IP Routing Parameters hierarchy and set the following:

- Routing Table Export = Once at End of Simulation. This asks the router to export its routing table at the end of the simulation to the simulation log.

- Click OK and then save your project.

#### Add the Remaining LANs:

- Highlight or select simultaneously (using shift and left-click) all five objects that you currently have in the project workspace (one router, two LANs, and two links). You can click-and-drag a box around the objects to do this.

- Press Ctri+C to copy the selected objects and then press Ctri+V to paste them.

- Repeat step 2 three times to generate three new copies of the objects and arrange them in a way similar to the following figure. Rename all objects as shown.

- Connect routers, as shown, using PPP_DS3 links.

The PPP_DS3 link has a data rate of 44.736 Mbps.

### Choose the Statistics

To test the performance of the RIP protocol, we will collect the following statistics:

- Right-click anywhere in the project workspace and select Choose Individual Statistics from the pop-up menu.

- In the Choose Results dialog box, check the following statistics:

  - Global Statistics  RIP  Traffic Sent (bits/sec).

  - Global Statistics  RIP  Traffic Recerved (bits/sec).

  - Nodes Statistics  Route Table  Total Number of Updates.

- Click OK and then save your project.

RlP traffic is the total amount of RIP update traffic (in bits) sent/received per second by all the nodes using RlP as the routing protocol in the IP interfaces in the node.

Total Number of Updates is the number of times the routing table at this node gets updated (e.g., due to a new route addition, an existing route deletion, and/or a next hop update).

### Configure the Simulation

Here we need to configure some of the simulation parameters:

- Click on Error! Objects cannot be created from editing field codes. and the Configure Simulation window should appear.

- Set the duration to be 10.0 minutes.

- Click on the Global Attributes tab and change the following attributes:

    - IP Dynamic Routing Protocol = RIP. This sets the RIP protocol to be the routing protocol of all routers in the network.

    - IP Interface Addressing Mode = Auto Addressed/Export.

    - RIP Sim Efficiency = Disabled. If this attribute is enabled, RIP will stop after the "RIP Stop Time" But we need the RIP to keep updating the routing table in case there is any change in the network (as we will see in the second scenario).

- Click OK and then save the project.

Auto Addressed means that all IP interfaces are assigned IP addresses automatically during simulations. The class of address (e.g., A, B, or C) is determined based on the number of hosts in the designed network. Subnet masks assigned to these interfaces are the default subnet masks for that class.

Export causes the auto-assigned lP interface to a file (name of the file is <net_name> ip_address.gdf and gets saved in the primary model directory).

### The Ping Scenario

In this scenario we will utilize the ping model to print the list of traversed nodes while the ICMP request message is sent to the destination and the ICMP response is received from the destination. Traversed routes are logged in the simulation log file …

- Select Duplicate Scenario from the Scenarios menu and name it ICMP_Ping  Click OK.

- Select both Router1 and Router3 simultaneously (click on both of them while holding the Shift key)  Select the Protocols menu  IP  Demands  Configure Ping Traffic on Selected Nodes.

- Change the Pattern attribute to Record Route as shown  Click OK.

Notice that a Ping Parameter node will be added to the project space. In addition the ping demand is created between Rotuer1 and Router3 as a dotted line.

### The Failure Scenario

The routers in the network we created will build their routing tables with no need to update these tables further because we didn't simulate any node or link failures. In this scenario we will simulate failures so that we can compare the behavior of the routers in both cases.

- Select Duplicate Scenario from the Scenarios menu and name it Failure  Click OK.

- Open Object Palette by clicking Error! Objects cannot be created from editing field codes.. Select the Utilities palette from the drop-down menu  Add a Failure Recovery object to your workspace and name it Failure as shown  Close the Object Palette dialog box.

- Right-click on the Failure object  Edit Attributes  Expand the Link Failure/Recovery Specification hierarchy  Set rows to 1  Set the attributes of the added row, row 0, as follows:

- This will "fail" the link between Router1 and Router2 200 seconds into the simulation.

- Click OK and then save the project.

### Run the Simulation

To run the simulation for both scenarios simultaneously:

- Go to the Scenarios menu  Select Manage Scenarios.

- Change the values under the Results column to <collect> (or <recollect>) for the three scenarios. Compare to the following figure.

- Click OK to run the three simulations. Depending on the speed of your processor, this may take several seconds to complete.

- After the three simulation runs complete, one for each scenario, click Close  Save your project.

### View the Results

#### Compare the Number of Updates:

- Select Compare Results from the Results menu.

- Change the drop-down menu in the right-lower part of the Compare Results dialog box to Stacked Statistics and Select Scenarios as shown.

- Select the Total Number of Updates statistic for Router1 and click Show  Select the NO_Failure and Failure scenarios in the Select Scenarios dialog box.

- You should get two graphs, one for each scenario. Right-click on each graph and select Draw Style  Bar.

- The resulting graphs should resemble the following (you can zoom in on the graphs by clicking-and-dragging a box over the region of interest):

#### Obtain the IP Addresses of the Interface:

Before checking the contents of the routing tables, we need to determine the IP address information for all interfaces in the current network. Recall that these IP addresses are assigned automatically during simulation, and we set the global attribute IP Interface Addressing Mode to export this information to a file.

- From the File menu choose Model Files  Refresh Model Directories. This causes OPNET IT Guru to search the model directories and update its list of files.

- From the File menu choose Open  From the drop-down menu choose Generic Data File  Select the <your initials>_RIP-NO_Failure-ip_addresses file (the other file created from the Failure scenario should contain the same information)  Click OK.

- The following is a part of the gdf file content. It shows the IP addresses assigned to the interfaces of Router1 in our network. For example the interface of Router1 that is connected to Net11 has the IP address 192.0.0.1 (Note: Your result may vary due to different nodes placement.) The Subnet Mask associated with that interface indicates that the address of the sub-network, to which the interface is connected, is 192.0.0.0 (i.e., the logical AND of the interface IP address and the subnet mask).

- Print out the layout of the network you implemented in this lab. On this layout, from the information included in the gdf file, write down the IP addresses associated with Router1 as well as the addresses assigned to each sub-network as shown in the following two figures (Note: Your IP addresses may vary due to different nodes placement.)

#### Getting the Ping Report:

- To check the content of the ping report for Router1:

  - Go to the ICMP_Ping scenario  Go to the Results menu  Open Simulation Log  Click on the field PING REPORT for "Campus Network Router1". The report should resemble the following one:

#### Compare the Routing Tables Content:

- To check the content of the routing tables in Router1 for the Failure and NO_Failure scenarios:

- Go to the Results menu.  Open Simulation Log  Expand the hierarchy on the left as shown below.  Click on the field COMMON ROUTE TABLE.

- Carry out the previous step for the Failure and NO_Failure scenarios. The following are partial contents of Router1's routing table for both scenarios (Note: Your results may vary due to different nodes placement):

Routing table of Router1 (NO Failure scenario):

Loopback interface allows a client and a server on the same host to communicate with each other using TCP/IP.

Routing table of Router1 (Failure scenario):

## Further readings

- RIP: IETF RFC number 2453 (www.ietf.org/rfc.html)

- ICMP. IETF RFC number 792 (www.ietf.org/rfc.html).

## Exercises

- Obtain and analyze the graphs that compare the sent RIP traffic for the Failure and NO_Failure scenarios. Make sure to change the draw style for the graphs to Bar.

- Describe and explain the effect of the failure of the link connecting Router1 to Router2 on the routing tables of Router1.

- Create another scenario as a duplicate of the Failure scenario. Name the new scenario Q3_Recover. In this new scenario have the link connecting Router1 to Router2 recover after 400 seconds (make sure to keep the failure that occurs at the 200th second). Generate and analyze the graph that shows the effect of this recovery on the Total Number of Updates in the routing table of Router1. Check the contents of Router1's routing table. Compare this table with the corresponding routing tables generated in the NO_Failure and Failure scenarios.

- Change the Ping packet size to 5000 bytes (Hint: Edit the attributes of the Ping Parameters node). Run the simulation to generate a new Ping report. What is the effect of the new size on the ICMP packet response time?

## Lab Report

Prepare a report that follows the guidelines explained in Lab 0. The report should include the answers to the above exercises as well as the graphs you generated from the simulation scenarios. Discuss the results you obtained and compare these results with your expectations. Mention any anomalies or unexplained behaviors.